350W 3in1 Moving head light

OPERATING INSTRUCTIONS：

(Please read this manual carefully before use）

Products Description:

350W 3in1 beam moving head light

350W 3in1 beam pattern light

Voltage：100-250V，50/60HZ

Power：600W

Bulb：350W YODN

Channel mode：16/24 Channels

Color Wheel：13 colors + white with rotating rainbow effect，half color or full color mode

Gobo：night rotation gobo, fifteen fix gobo.

Prism：Two Prism 16 prisms +row of mirror（16棱镜加排镜）

Scanning：Level scanning540°,Vertical scanning270°,16bit accurate scan，Electronic correction.

Optical Lens：3in1 high accurate Optical Lens

Focus: electronic focus

Dimmer：0%~100% linear dimmer

Frost ：0%~100%linear frost

Flash：双侧刀频闪率最高可以达到次每秒，并可先择随机频闪及脉冲频闪。

Display：Color touch screen

| Instructions |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | DMX Address | 001-512 |  |  |
| 2 | Sys set | DMX Mode | Simple mode 16CH |  |
|  |  |  | Standard mode 24CH |  |
|  |  | Pan Inverse | No |  |
|  |  |  | OFF |  |
|  |  | Tilt Inverse | No |  |
|  |  |  | OFF |  |
|  |  | XY Biackout | No |  |
|  |  |  | OFF |  |
|  |  | Run mode | DMX |  |
|  |  |  | Auto |  |
|  |  | Backlight | 60S |  |
|  |  |  | Always |  |
|  |  | Save |  |  |
|  |  | Exit |  |  |
| 3 | Manual | Pan | 0-255 |  |
|  |  | Pan Fine | 0-255 |  |
|  |  | Tilt | 0-255 |  |
|  |  | Tilt Fine | 0-255 |  |
|  |  | Pan Tilt Speed | 0-255 |  |
|  |  | function | 0-255 |  |
|  |  | color | 0-255 |  |
|  |  | color Fine | 0-255 |  |
|  |  | Effect Speed | 0-255 |  |
|  |  | Static Gobo | 0-255 |  |
|  |  | Rotating Gobo | 0-255 |  |
|  |  | Rot Gobo | 0-255 |  |
|  |  | Rot Gobo Fine | 0-255 |  |
|  |  | Prism | 0-255 |  |
|  |  | Prism Rot | 0-255 |  |
|  |  | Frost | 0-255 |  |
|  |  | Zoom | 0-255 |  |
|  |  | Zoom Fine | 0-255 |  |
|  |  | Focus | 0-255 |  |
|  |  | Focus Fine | 0-255 |  |
|  |  | Auto Focus | 0-255 |  |
|  |  | Shutter | 0-255 |  |
|  |  | Dimmer | 0-255 |  |
|  |  | Dimmer Fine | 0-255 |  |
|  |  | Exit |  |  |
| 4 | Infor |  |  |  |
| 5 | Factory | Passwords 158 |  |  |
| 6 | Lamp | Lamp Status |  | OFF |
|  |  |  |  | ON |
|  |  | Lamp on power |  | OFF |
|  |  |  |  | ON |
|  |  | EXIT |  |  |
| 7 | Reset | Reset pan&Tilt |  |  |
|  |  | Reset Effect |  |  |
|  |  | Reset All |  |  |
|  |  | EXIT |  |  |
| 8 | Disolay | Language |  | Chinese |
|  |  |  |  | English |
|  |  | Disolay flip |  | YES |
|  |  |  |  | NO |
|  |  | touch calibration |  |  |
|  |  | EXIT |  |  |

| B350 M2Control board (channel table) |  |  |  |
| --- | --- | --- | --- |
| Standard mode | Simple mode | DMX Value | Function |
| 1 | 1 | 0－255 | Pan |
| 2 | * | 0－255 | Pan Low |
| 3 | 2 | 0－255 | Tilt |
| 4 | * | 0－255 | Tilt Low |
| 5 | 3 | 0－255 | P/T Speed |
| 6 | 4 |  | Macro function |
|  |  | 0－19 | Reserve |
|  |  | 20－24 | 230W power |
|  |  | 25-29 | 350W power |
|  |  | 30-49 | Reserve |
|  |  | 50-59 | Reserve |
|  |  | 60-69 | Reserve |
|  |  | 70-79 | P/T Rotate Off Light |
|  |  | 80-89 | P/T Rotate On Light |
|  |  | 90-99 | Reserve |
|  |  | 100-109 | Reserve |
|  |  | 110-119 | Reserve |
|  |  | 120-129 | Reserve |
|  |  | 130－139 | Lamp |
|  |  | 140－149 | P/T Reset |
|  |  | 150－189 | Effect Reset |
|  |  | 190－199 | Reserve |
|  |  | 200－209 | whole Reset |
|  |  | 210－229 | Reserve |
|  |  | 230－239 | Lamp Off |
|  |  | 240－255 | Reserve |
| 7 | 5 |  | Macro Colour |
|  |  | 0-127 | Linear Colour |
|  |  | 128-129 | white |
|  |  | 130-134 | Colour 1 |
|  |  | 135-138 | Colour 2 |
|  |  | 139-143 | Colour 3 |
|  |  | 144-147 | Colour 4 |
|  |  | 148-152 | Colour 5 |
|  |  | 153-157 | Colour 6 |
|  |  | 158-161 | Colour 7 |
|  |  | 162-166 | Colour 8 |
|  |  | 167-171 | Colour 9 |
|  |  | 172-176 | Colour 10 |
|  |  | 177-180 | Colour 11 |
|  |  | 181-185 | Colour 12 |
|  |  | 186-189 | Colour 13 |
|  |  | 190-220 | FAST ROTATION - SLOW ROTATION |
|  |  | 221-224 | STOP |
|  |  | 225-255 | SLOW ROTATION - FAST ROTATION |
| 8 | * | 0-255 | Reserve |
| 9 | 6 |  | Gobo 1 |
|  |  | 0-3 | white |
|  |  | 4月9日 | Gobo 1 |
|  |  | 10月15日 | Gobo 2 |
|  |  | 16-21 | Gobo 3 |
|  |  | 22-27 | Gobo 4 |
|  |  | 28-33 | Gobo 5 |
|  |  | 34-39 | Gobo 6 |
|  |  | 40-45 | Gobo 7 |
|  |  | 46-51 | Gobo 8 |
|  |  | 52-57 | Gobo 9 |
|  |  | 58-63 | Gobo 10 |
|  |  | 64-69 | Gobo 11 |
|  |  | 70-75 | Gobo 12 |
|  |  | 76-81 | Gobo 13 |
|  |  | 82-87 | Gobo 14 |
|  |  | 88-95 | Gobo 1 Jitter |
|  |  | 96-103 | Gobo 2 Jitter |
|  |  | 104-111 | Gobo 3 Jitter |
|  |  | 112-119 | Gobo 4 Jitter |
|  |  | 120-127 | Gobo 5 Jitter |
|  |  | 128-135 | Gobo 6 Jitter |
|  |  | 136-143 | Gobo 7 Jitter |
|  |  | 144-151 | Gobo 8 Jitter |
|  |  | 152-159 | Gobo 9 Jitter |
|  |  | 160-167 | Gobo 10 Jitter |
|  |  | 168-175 | Gobo 11 Jitter |
|  |  | 176-183 | Gobo 12 Jitter |
|  |  | 184-191 | Gobo 13 Jitter |
|  |  | 192-199 | Gobo 14 Jitter |
|  |  | 200-201 | white |
|  |  | 202-227 | FAST ROTATION - SLOW ROTATION |
|  |  | 228-229 | STOP |
|  |  | 230-255 | SLOW ROTATION - FAST ROTATION |
| 10 | 7 |  | Gobo 2 |
|  |  | 0 | S white |
|  |  | 1月4日 | L white |
|  |  | 5月8日 | Gobo 2 -1 |
|  |  | 9月12日 | Gobo 2 -2 |
|  |  | 13-16 | Gobo 2 -3 |
|  |  | 17-20 | Gobo 2 -4 |
|  |  | 21-24 | Gobo 2 -5 |
|  |  | 25-28 | Gobo 2 -6 |
|  |  | 29-32 | Gobo 2 -7 |
|  |  | 33-36 | Gobo 2 -8 |
|  |  | 37-40 | Gobo 2 -9 |
|  |  | 41-48 | Gobo 2 -1 Jitter |
|  |  | 49-56 | Gobo 2 -2 Jitter |
|  |  | 57-64 | Gobo 2 -3 Jitter |
|  |  | 65-72 | Gobo 2 -4 Jitter |
|  |  | 73-80 | Gobo 2 -5 Jitter |
|  |  | 81-88 | Gobo 2 -6 Jitter |
|  |  | 89-96 | Gobo 2 -7 Jitter |
|  |  | 97-104 | Gobo 2 -8 Jitter |
|  |  | 105-112 | Gobo 2 -9 Jitter |
|  |  | 113-119 | L white |
|  |  | 120-185 | FAST ROTATION - SLOW ROTATION |
|  |  | 186-189 | STOP |
|  |  | 190-255 | SLOW ROTATION - FAST ROTATION |
| 11 | 8 |  | Gobo2/Rot |
|  |  | 0-127 | Rotation |
|  |  | 128-190 | FAST ROTATION - SLOW ROTATION |
|  |  | 191-192 | STOP |
|  |  | 193-255 | SLOW ROTATION - FAST ROTATION |
| 12 | * | 0-255 | Reserve |
| 13 | 9 |  | Prism |
|  |  | 0-63 | empty |
|  |  | 64-127 | Prism-1 |
|  |  | 128-191 | Prism-2 |
|  |  | 192-255 | Prism-1/Prism-2 |
| 14 | 10 |  | Prism-1/Rot |
|  |  | 0－127 | Rotation |
|  |  | 128－189 | FAST ROTATION - SLOW ROTATION |
|  |  | 190－192 | STOP |
|  |  | 193－255 | SLOW ROTATION - FAST ROTATION |
| 15 | 11 |  | Prism-2/Rot |
|  |  | 0－127 | Rotation |
|  |  | 128－189 | FAST ROTATION - SLOW ROTATION |
|  |  | 190－192 | STOP |
|  |  | 193－255 | SLOW ROTATION - FAST ROTATION |
| 16 | 12 |  | Frost |
|  |  | 0－64 | Reserve |
|  |  | 65－255 | Linear Frost |
| 17 | 13 |  | Zoom |
|  |  | 0－255 | Linear Zoom |
| 18 | * | 0－255 | Zoom Fine |
| 19 | 14 | 0－255 | Focus |
| 20 | * | 0－255 | Zoom Low |
| 21 | * | 0－255 | Reserve |
| 22 | 15 |  | Shutter |
|  |  | 0-31 | Closed |
|  |  | 32-63 | Open |
|  |  | 64-95 | SLOW - FAST |
|  |  | 96-127 | Open |
|  |  | 128-143 | FAST STROBE-SLOW STROBE |
|  |  | 144-159 | SLOW STROBE - FAST STROBE |
|  |  | 160-191 | Open |
|  |  | 192-223 | RANDOM SLOW - FAST |
|  |  | 224-255 | Open |
| 23 | 16 |  | Dimmer |
|  |  | 0－255 | Linear Dimmer |
| 24 | * |  | Reserve |

| Channel | Channel Mode |  |
| --- | --- | --- |
|  | 18 | 22 |
| 1 | Color Wheel | Color Wheel |
| 2 | 切光/频闪 | 切光/频闪 |
| 3 | Dimmer | Dimmer |
| 4 | fix Gobo wheel | fix Gobo wheel |
| 5 | Rotation Gobo wheel | Rotation Gobo wheel |
| 6 | Gobo Rot | Gobo Rot |
| 7 | Frost | Frost |
| 8 | Prism | Prims |
| 9 | Prism Rotation 1 | Prism Rotation 1 |
| 10 | Prism= 2 | Prism 2 |
| 11 | Prism Rotation 2 | Prism Rotation 2 |
| 12 | Zoom | Zoom |
| 13 | Focus | Focus |
| 14 | X | X |
| 15 | Pan Low | Pan Low |
| 16 | Y | Y |
| 17 | Tilt Low | Tilt Low |
| 18 | Bulb Control &Reset | Bulb Contol &Reset |
| 19 | Reserve | XY Speed |
| 20 | Reserve | Color wheel Speed |
| 21 | Reserve | Dimmer-Prism-Frost Speed |
| 22 | Reserve | Gobo wheel Speed |